# FLARM USB Updater

Hardware Design Document

REV A

PCB: 53.4 × 30.5 mm (2102 × 1200 mil), 2-layer FR4

March 2026

# 1. Overview

The FLARM USB Updater is a USB-to-RS232 adapter with an integrated 12V boost converter, designed to update FLARM firmware and FLARM displays without requiring external power. It connects to a PC via USB-C and to the FLARM unit via an RJ45 cable following the IGC specification.

## 1.1 Key Features

- USB-C powered — no external 12V supply needed

- FT232RL USB-UART bridge (uses built-in Windows usbser.sys driver)

- MAX3232 RS232 level converter for FLARM communication

- MT3608 boost converter: 5V USB → 12V FLARM power

- AMS1117-3.3 LDO for FLARM 3.3V pin

- USBLC6-2SC6 ESD protection on USB data lines

- 3 status LEDs: power (green), TX (green), RX (yellow)

- TTL UART breakout header (J3) for direct TTL access

- All SMT, JLCPCB assembly compatible

## 1.2 Use Cases

- FLARM firmware updates (straight RJ45 cable)

- FLARM display updates (crossover RJ45 cable, pins 5+6 swapped)

- IGC flight log download

- FLARM configuration and live NMEA diagnostics

- General-purpose USB to RS232 adapter (cable with GND/TX/RX only)

# 2. System Architecture

The design consists of five functional blocks:

## 2.1 Power Supply

Boost Converter (MT3608): Converts 5V USB to ~12.36V. Feedback divider R1 (100kΩ) / R2 (5.1kΩ) sets Vout = 0.6 × (1 + R1/R2) = 12.36V. Schottky diode D1 (B5819W) drops ~0.36V, yielding ~12.0V at V12_OUT. L1 is a 22µH inductor (top of MT3608 recommended range) chosen for lower peak current and ripple at our light load (~85mA).

LDO (AMS1117-3.3): Provides 3.3V from USB 5V for FLARM pin 3. R6 (10Ω) limits current between the LDO output and FLARM's internal 3.3V regulator to prevent backfeed.

Power Budget: Classic FLARM draws ~85mA at 12V. Total USB current: ~280mA, well within any USB port's capability.

## 2.2 USB Interface

USB-C connector (J1) with CC1/CC2 pull-down resistors (R4, R5: 5.1kΩ) for UFP sink identification. USBLC6-2SC6 (D2) provides ESD protection on D+/D- lines. Both USB-C orientations are supported.

## 2.3 USB-UART Bridge

FT232RL (U1) converts USB to TTL UART. Chosen over CH340C because the November 2024 Windows Update broke CH340 drivers; FT232RL uses the built-in usbser.sys driver. CBUS0 (TXLED#) and CBUS1 (RXLED#) drive status LEDs via active-low outputs. Pin 26 (TEST) tied to GND. Unused modem control inputs (RI#, DSR#, DCD#, CTS#) tied to GND.

## 2.4 RS232 Level Converter

MAX3232 (U2) converts TTL UART to RS232 levels using charge pump capacitors C5-C9 (all 100nF). Channel 1 is used for FLARM communication. Channel 2 is unused; T2IN (pin 10) must be tied to GND to prevent floating input oscillation.

## 2.5 LED Indicators

All LEDs use the same topology: VUSB → LED anode (pin 1) → LED cathode (pin 2) → R → sink.

- LED1 (green, KT-0805G): Power indicator. R7 (680Ω) to GND. Always on. I = (5-2.8)/680 = 3.2mA.

- LED2 (green, KT-0805G): TX activity. R8 (680Ω) to TXLED (CBUS0). I = (5-2.8)/680 = 3.2mA when active.

- LED3 (yellow, KT-0805Y): RX activity. R9 (680Ω) to RXLED (CBUS1). I = (5-2.1)/680 = 4.3mA when active.

# 3. RJ45 Pinout (IGC Specification)

Pin numbering follows the IGC GNSS FR specification. The SMD RJ45 connector (RJ-064-1) has reversed pad numbering (pad 1 = RJ45 pin 8), accounted for in the PCB routing.

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | +12V (V12_OUT) | Linked with pin 2 |
| 2 | +12V (V12_OUT) | Linked with pin 1 |
| 3 | +3.3V (V3V3_OUT via R6) | FLARM supplies 3V during updates |
| 4 | GND | Linked with pins 7, 8 |
| 5 | FLARM TX (RS232_RX) | PC-side: SUB-D9 pin 2 |
| 6 | FLARM RX (RS232_TX) | PC-side: SUB-D9 pin 3 |
| 7 | GND | Linked with pins 4, 8 |
| 8 | GND | PC-side: SUB-D9 pin 5 |

# 4. Net Names

| Net Name | Description |
| --- | --- |
| VUSB | USB 5V supply from USB-C |
| GND | Ground |
| VBOOST | Boost converter output (~12.36V) |
| V12_OUT | 12V after Schottky diode (~12.0V) |
| V3V3 | 3.3V LDO output |
| V3V3_OUT | 3.3V to FLARM via R6 (10Ω) |
| UD+ | USB data positive |
| UD- | USB data negative |
| CC1 | USB-C configuration channel 1 |
| CC2 | USB-C configuration channel 2 |
| UART_TX | TTL-level TX (FT232RL to MAX3232) |
| UART_RX | TTL-level RX (MAX3232 to FT232RL) |
| RS232_TX | RS232-level TX (to FLARM RX) |
| RS232_RX | RS232-level RX (from FLARM TX) |
| TXLED | FT232RL CBUS0 (active-low TX indicator) |
| RXLED | FT232RL CBUS1 (active-low RX indicator) |
| SW | Boost converter switch node (L1 to U3) |
| FB | Boost feedback divider midpoint |
| EN | Boost converter enable (pulled high via R3) |
| FT_3V3 | FT232RL internal 3.3V output (C12 decoupling) |

# 5. Bill of Materials

38 SMT components. 7 Extended parts (× $3 = $21 setup fee). J3 is unpopulated (hand-solder when needed).

Estimated cost: 5 boards ≈ $16.70/ea, 10 boards ≈ $12.30/ea

| Ref | Part | Description | Package | LCSC | Class |
| --- | --- | --- | --- | --- | --- |
| U1 | FT232RL-REEL | USB to UART converter | SSOP-28 | C8690 | Extended |
| U2 | MAX3232CSE+T | RS232 transceiver (dual) | SOIC-16 | C7258 | Extended |
| U3 | MT3608 | Step-up DC-DC converter | SOT-23-6 | C84817 | Extended |
| U4 | AMS1117-3.3 | 3.3V LDO regulator | SOT-223 | C6186 | Basic |
| D1 | B5819W SL | Schottky diode (boost) | SOD-123 | C8598 | Basic |
| D2 | USBLC6-2SC6 | USB ESD protection | SOT-23-6 | C7519 | Extended |
| L1 | SMNR4020-22UH | 22µH inductor (boost) | 4x4mm SMD | C135264 | Extended |
| J1 | TYPE-C-31-M-12 | USB-C receptacle | SMD 12-pin | C165948 | Extended |
| RJ1 | RJ-064-1 | RJ45 jack (SMD) | SMD | TBD | Extended |
| J3 | HDR-M 1x3P | TTL UART breakout (unpop.) | 2.54mm PTH | N/A | N/A |
| LED1 | KT-0805G | Power LED (green) | 0805 | C2297 | Basic |
| LED2 | KT-0805G | TX LED (green) | 0805 | C2297 | Basic |
| LED3 | KT-0805Y | RX LED (yellow) | 0805 | C2296 | Basic |
| R1 | 0402WGF1003TCE | 100kΩ ±1% (boost FB top) | 0402 | C25741 | Basic |
| R2 | 0402WGF5101TCE | 5.1kΩ ±1% (boost FB bottom) | 0402 | C25905 | Basic |
| R3 | 0402WGF1002TCE | 10kΩ ±1% (boost EN pull-up) | 0402 | C25744 | Basic |
| R4 | 0402WGF5101TCE | 5.1kΩ ±1% (USB CC1) | 0402 | C25905 | Basic |
| R5 | 0402WGF5101TCE | 5.1kΩ ±1% (USB CC2) | 0402 | C25905 | Basic |
| R6 | 0402WGF100JTCE | 10Ω ±1% (LDO to RJ45) | 0402 | C25077 | Basic |
| R7 | 0402WGF6800TCE | 680Ω ±1% (power LED) | 0402 | C25130 | Basic |
| R8 | 0402WGF6800TCE | 680Ω ±1% (TX LED) | 0402 | C25130 | Basic |
| R9 | 0402WGF6800TCE | 680Ω ±1% (RX LED) | 0402 | C25130 | Basic |
| C1 | CL21A226MAQNNNE | 22µF 25V (boost input) | 0805 | C45783 | Basic |
| C2 | CL05B104KO5NNNC | 100nF (boost input HF) | 0402 | C307331 | Basic |
| C3 | CL21A226MAQNNNE | 22µF 25V (boost output) | 0805 | C45783 | Basic |
| C4 | CL21A475KAQNNNE | 4.7µF (boost output HF) | 0805 | C1779 | Basic |
| C5-C9 | CL05B104KO5NNNC | 100nF (MAX3232 charge pump) | 0402 | C307331 | Basic |
| C10 | CL21A475KAQNNNE | 4.7µF (FT232RL VCC) | 0805 | C1779 | Basic |
| C11-C14 | CL05B104KO5NNNC | 100nF (FT232RL decoupling) | 0402 | C307331 | Basic |
| C15 | CL21A226MAQNNNE | 22µF 25V (LDO input) | 0805 | C45783 | Basic |
| C16 | CL21A226MAQNNNE | 22µF 25V (LDO output) | 0805 | C45783 | Basic |

# 6. PCB Design

Board: 53.4 × 30.5 mm (2102 × 1200 mil), 2-layer FR4, 1oz copper.

- GND copper pour on both layers (164 poured regions)

- 51 vias (49 GND, 2 VUSB) for ground plane stitching

- Power traces (VUSB, VBOOST, SW, V3V3): 20mil

- V12_OUT traces: 15mil

- Signal traces (UART, RS232, USB, LED): 10mil

- USB differential pair (UD+/UD-): length matched at 24.3mm

- 312 trace segments across 29 routed nets

- Designed in EasyEDA Pro, manufactured by JLCPCB

# 7. Key Design Decisions

### FT232RL over CH340C

November 2024 Windows Update broke CH340 drivers. FT232RL uses the built-in usbser.sys driver, eliminating driver issues.

### USBLC6-2SC6 ESD Protection

Aviation and workshop environments have significant ESD risk. The USBLC6-2SC6 protects USB data lines with minimal capacitance impact.

### 22µH Inductor

Top of the MT3608 recommended range (4.7-22µH). Chosen for lower peak inductor current and reduced output ripple at our light load (~85mA at 12V). The SMNR4020-22UH (4x4mm) fits the board.

### R6 Current Limiter (10Ω)

FLARM outputs up to 200mA at 3.3V on pin 3 (per FLARM manual). Without R6, two LDO outputs would fight, causing uncontrolled current flow. R6 limits mismatch current to ~10mA max. AMS1117 handles backfeed safely with 22µF caps (no protection diode needed). Can be replaced with 0Ω if display voltage drop is an issue.

### 680Ω LED Resistors

KT-0805G (green) has a maximum forward current of 5mA. With 680Ω: I = (5-2.8)/680 = 3.2mA, providing comfortable margin. Yellow LEDs run at 4.3mA. Both values produce adequate brightness (430mcd green, 175mcd yellow).

# 8. Design Verification (REV A)

Full electrical review completed 2026-03-13:

- FT232RL: all 28 pins verified (TEST→GND, CTS/DSR/DCD/RI→GND, OSCI/OSCO NC)

- MAX3232: charge pump caps correct, ch1 UART↔RS232, T2IN (pin 10) tied to GND

- MT3608: boost topology correct, Vout=12.36V, D1 polarity correct

- USB-C: CC pulldowns 5.1kΩ, D+/D- through USBLC6, both orientations

- AMS1117: V3V3 → R6 (10Ω) → RJ45 pin 3, adequate headroom (1.7V)

- LED circuits: active-low CBUS, correct polarity, 3.2–4.3mA

- RJ45 pinout matches FLARM IGC specification

- USB D+/D- length matched (24.3mm each)

- Power budget: ~280mA within USB 2.0 500mA limit

- All capacitor voltage ratings ≥1.8×, all resistor power ratings <40%

# 9. Remaining Items

- Run ERC and DRC in EasyEDA Pro

- Verify SMD RJ45 (RJ-064-1) C-number for JLCPCB

- Export Gerber, BOM, CPL for JLCPCB order

- Test AMS1117 output stability with MLCC caps after build

- Optional: Program FT232RL EEPROM via FT_PROG (device description, serial number)